INSTRUCTIONS

FOR

GSD95 GEOID MODEL

INSTALLATION AND OPERATION

IN

LGO

August 14, 2004

The GSD95 Geoid Model is available from Geomatics Canada to determine Geoid Separations between the GRS 1980 geocentric ellipsoid based on the NAD-83 datum and the equipotential surface representing the global mean sea level. The GSD95 model provides a typical relative accuracy of a few centimetres + 1ppm between 2 or more points.

The GSD95 geoid model must be used with the GSD95.BIN coefficient and GSD95.SLV grid limit files together with the GSD95.EXE program file. Additional information about the GSD95 Geoid Model can be obtained from the Geodetic Survey Division (GSD) of Geomatics Canada (GPS-H V2.0 Package) at the following address:

Information Services            Tel:(613) 995-4410

Geodetic Survey Division     Fax:(613) 995-3215

615 Booth Street                 E-mail: information @geod.nrcan.gc.ca

Ottawa, Ontario                   Internet: http://www.geod.nrcan.gc.ca

K1A 0E9                             GOPHER:gopher.nrcan.gc.ca

The GSD95 Geoid Model is installed by copying the GSD95.BIN and the GSD95.SLV files together with the GSD95.EXE program file in any subdirectory. A new Geoid Model must be defined in LGO Coordinate Systems Management Group by creating the GSD95 Model using GRS 1980 Ellipsoid, Geodetic Coordinates and locating the GSD95.EXE program name and its subfolder on the PC as per Figure 1.

# Figure 1: GSD95 Definition in LGO

Following the proper definition of the GSD95 Geoid Model in LGO, the GSD95 Geoid Model must then be specified in new or existing Coordinate Systems based on the same Ellipsoid (GRS 1980) as per Figure 2.

# Figure 2: GSD95 Attachment in a LGO Coordinate System

Once the GSD95 Geoid Model is correctly attached to one or more Coordinate Systems, it can be used to compute Geoid Separations in LGO Projects based on these Coordinate Systems. Geoid Separation calculations are initiated by selecting Compute Geoid Separations from the Tools pull down menu. If the local Latitude and/or Longitude of Points are outside of the GSD95 coefficient file limits, an error message will be presented saying that Geoid Separation has failed. Confirmation of the proper calculation of Geoid Separations will be presented so that Orthometric Heights can be displayed together with local Ellipsoid Heights as per Figure 3.

Figure 3: Confirmation of proper Geoid Separation Calculations in a LGO Project