INSTRUCTIONS

FOR

HT1_01 GEOID MODEL

INSTALLATION AND OPERATION

IN

LGO

August 14, 2004

The HT1_01 Geoid Model is available from Geomatics Canada to determine Geoid Separations between the GRS 1980 geocentric ellipsoid based on the NAD-83 (CSRS98) datum and CGVD28 vertical datum. The HT1_01 model is based on the Canadian Geoid95 Model with additional space-based Vertical Transformations to produce new (absolute) Geoid Separations between the two systems. Its “absolute” accuracy is typically +/- 5 cm in Southern Regions of Canada and about a few decimetres in Northern Regions. Its “relative” accuracy is typically a few centimetres + 1 ppm between 2 or more points. It is therefore possible to obtain GPS-derived Orthometric Heights compatible with the Canadian Vertical Bench Mark system (CGVD) surrounding the area.

The HT1_01 geoid model must be used with the HT1_01.BIN coefficient and HT1_01.SLV grid limit files together with the HT1_01.EXE program file. Additional information about the HT1_01 Geoid Model can be obtained from the Geodetic Survey Division (GSD) of Geomatics Canada (GPS-H V2.0 Package) at the following address:

Information Services            Tel:(613) 995-4410

Geodetic Survey Division     Fax:(613) 995-3215

615 Booth Street                 E-mail: information @geod.nrcan.gc.ca

Ottawa, Ontario                   Internet: http://www.geod.nrcan.gc.ca

K1A 0E9                             GOPHER:gopher.nrcan.gc.ca

The HT1_01 Geoid Model is installed by copying the HT1_01.BIN and the HT1_01.SLV files together with the HT1_01.EXE program file in any subdirectory. A new Geoid Model must be defined in LGO Coordinate Systems Management Group by creating the HT1_01 Model using GRS 1980 Ellipsoid, Geodetic Coordinates and locating the HT1_01.EXE program name and its subfolder on the PC as per Figure 1.

# Figure 1: HT1_01 Definition in LGO

Following the proper definition of the HT1_01 Geoid Model in LGO, the HT1_01 Geoid Model must then be specified in new or existing Coordinate Systems based on the same Ellipsoid (GRS 1980) as per Figure 2.

# Figure 2: HT1_01 Attachment in a LGO Coordinate System

Once the HT1_01 Geoid Model is correctly attached to one or more Coordinate Systems, it can be used to compute Geoid Separations in LGO Projects based on these Coordinate Systems. Geoid Separation calculations are initiated by selecting Compute Geoid Separations from the Tools pull down menu. If the local Latitude and/or Longitude of Points are outside of the HT1_01 coefficient file limits, an error message will be presented saying that Geoid Separation has failed. Confirmation of the proper calculation of Geoid Separations will be presented so that Orthometric Heights can be displayed together with local Ellipsoid Heights as per Figure 3.

Figure 3: Confirmation of proper Geoid Separation Calculations in a LGO Project