GEOID96 for Leica Geo Office

GEOID96 for LGO

February, 2004

Leica Geosystems Incorporated

1. INTRODUCTION:

The Geoid96 model can be used in Leica Geo office (LGO) for geoid undulation calculations based on NAD-83 Latitudes and Longitudes. The Geoid96 model consists of a single program, named GEOID96.EXE, with one or more grid coefficient file(s), named geo96*.geo, corresponding to different areas of continental US, Alaska, Hawaii, Puerto Rico, Virgin Islands and Caribbean territories. Position limits of the 6 different continental US, 2 Alaska, 2 Caribbean, 1 Hawaii and 1 Puerto-Rico/Virgin Islands areas are listed below:

GEO96NE.GEO	the GEOID96 grid for the Northeastern U.S.	  36-50N,  89- 66W

GEO96NC.GEO   	the GEOID96 grid for the Northcentral U.S.		  36-50N, 107- 84W

GEO96NW.GEO	the GEOID96 grid for the Northwestern U.S.	  36-50N, 125-102W

GEO96SE.GEO	the GEOID96 grid for the Southeastern U.S.	  24-38N,  89- 66W

GEO96SC.GEO	the GEOID96 grid for the Southcentral U.S.		  24-38N, 107- 84W

GEO96SW.GEO	 the GEOID96 grid for the Southwestern U.S.	  24-38N, 125-102W

GEO96AN.GEO	 the GEOID96 grid for North Alaska 		  60-72N, 179-128W

GEO96AS.GEO	 the GEOID96 grid for South Alaska		  51-63N, 179-128W

GEO96HW.GEO	 the GEOID96 grid for the Principal Hawaiian Islands 18-24N, 161-154W

GEO96PR.GEO	 the GEOID96 grid for Puerto Rico - Virgin Islands     15-21N, 68-64W

GEO96CW.GEO	 the GEOID96 grid for West – Caribbean                       9-28N, 86-71W

GEO96CE.GEO	 the GEOID96 grid for East – Caribbean                        9-28N, 73-58W

It is under the user’s responsibility to obtain the right number and the adequate Geoid96 Grid coefficient (geo96*.geo) file(s) for the required territory(ies). A maximum of 14 Grid coefficient files can be opened by the program. It is expected that some of the Geoid96 grid coefficient files will be updated from time to time. Additional information about Geoid96 model can be found in the readme.txt file contained in GEOID96.ZIP.

2. INSTALLATION and OPERATION in LGO:

Decompress (pkunzip) GEOID96.ZIP file in any subdirectory existing on the current hard disk drive (C:\GEOID96 as an example). The GEOID96.ZIP file should contain the following files: GEOID96.EXE, GEOID96.DOC and one or more geo96*.geo grid coefficient files. Make sure that there is at least ONE Grid Coefficient file (geo96*.geo) in that same subdirectory corresponding to your area.

The Geoid Model is installed by copying the GEOID96EXE program file with one or more geo96*.geo file(s) in any subdirectory. A new Geoid Model must be defined in LGO Coordinate Systems Management Group by creating the GEOID96 Model using GRS 1980 Ellipsoid, Geodetic Coordinates and locating the GEOID96.EXE program name and its subfolder on the PC as per Figure 1.

Figure 1: Definition of GEOID96 Model in LGO

Following the proper definition of the GEOID96 Geoid Model in LGO, the GEOID96 Geoid Model must then be specified in new or existing Coordinate Systems based on the same Ellipsoid (GRS 1980) as per Figure 2.

# Figure 2: GEOID96 Attachment in a LGO Coordinate System

Once the GEOID96 Geoid Model is correctly attached to one or more Coordinate Systems, it can be used to compute Geoid Separations in LGO Projects based on these Coordinate Systems. Geoid Separation calculations are initiated by selecting Compute Geoid Separations from the Tools pull down menu. If the local Latitude and/or Longitude of Points are outside of the GEOID96 coefficient files limits, an error message will be presented saying that Geoid Separation has failed. Confirmation of the proper calculation of Geoid Separations will be presented so that Orthometric Heights can be displayed together with local Ellipsoid Heights as per Figure 3.

Figure 3: Confirmation of proper Geoid Separation Calculations in LGO