INSTRUCTIONS

FOR

Geoid99_27 GEOID MODEL

INSTALLATION AND OPERATION

IN

LGO

August 14, 2004

The Geoid99_27 model can be used in LGO to compute NAD27 geoid separations with respect to the Clarke 1866 ellipsoid and NAVD88 vertical datum. The Geoid99_27 model is based on the Geoid99 model with the appropriate shifts in heights derived from the average datum shifts between NAD83 and NAD27 datums: –10.0 m, 160.0 m and 180.0 m in Cartesian X, Y and Z respectively and the difference between the GRS1980 and the Clarke 1866 ellipsoids. The relative accuracy of the Geoid99_27 model is typically 1cm + 1 ppm between 2 or more points.

The Geoid99_27 geoid model must be used with the G1999*.BIN coefficient files together with the GEOID99_27.EXE program file. Additional information about the Geoid99_27 model can be found in the Local-Geoids.doc file.

The Geoid99_27 Geoid Model is installed by copying the G1999*.BIN coefficient files together with the GEOID99_27.EXE program file in any subdirectory. A new Geoid Model must be defined in LGO Coordinate Systems Management Group by creating the Geoid99_27 Model using Clarke 1866 Ellipsoid, Geodetic Coordinates and locating the GEOID99_27.EXE program name and its subfolder on the PC as per Figure 1.

# Figure 1: GEOID99_27 Definition in LGO

Following the proper definition of the GEOID99_27 Geoid Model in LGO, the GEOID99_27 Geoid Model must then be specified in new or existing Coordinate Systems based on the same Ellipsoid (Clarke 1866) as per Figure 2.

# Figure 2: GEOID99_27 Attachment in a LGO Coordinate System

Once the GEOID99_27 Geoid Model is correctly attached to one or more Coordinate Systems, it can be used to compute Geoid Separations in LGO Projects based on these Coordinate Systems. Geoid Separation calculations are initiated by selecting Compute Geoid Separations from the Tools pull down menu. If the local Latitude and/or Longitude of Points are outside of the GEOID99_27 coefficient file limits, an error message will be presented saying that Geoid Separation has failed. Confirmation of the proper calculation of Geoid Separations will be presented so that Orthometric Heights can be displayed together with local Ellipsoid Heights as per Figure 3.

Figure 3: Confirmation of proper Geoid Separation Calculations in a LGO Project