Quick Guide for

# SR20 Post-Processing LGO Script

The SR20 LGO Script is a three-part process.  The first step is to import data (typically from the CF card), create a project and attach a coordinate system (if required.)  When the user selects “Create Project” the tasks from Step 1 are executed and the user is prompted to go to Step 2.

The second step is an optional step for importing additional SR20 raw data.  If the user imported data from their first CF card in Step 1, they could now import data from the second CF in Step 2.  If more receivers are used, Step 2 can be repeated as many times as needed until all data is imported.  Once all data is imported, press “Continue” to move onto Step 3.

Step 3 allows the user to set a control point before starting the processing.  Here, the user types in the value of the control point in Northing, Easting, Ellipsoid height or Latitude, Longitude, Ellipsoid height.  Pressing “Process” begins the processing.  The point set as the control is held fixed and all other points are processed in reference to this point using the auto-processing settings in the LGO.

When the processing is complete, a dialog appears enabling the user to export an ASCII file.  This is the same dialog a user would normally see in LGO, including all the export options under the “Settings” button.

This message appears telling the user processing is complete. An ASCII file with the post-processed results is successfully created.