INSTRUCTIONS

FOR

GSD91 GEOID MODEL

INSTALLATION AND OPERATION

IN

LGO

August 14, 2004

The GSD91 Geoid Model is available from Geomatics Canada to determine Geoid Separations between the GRS 1980 geocentric ellipsoid based on the NAD-83 datum and the equipotential surface representing the global mean sea level. The GSD91 model provides a typical relative accuracy of a few centimetres + 1ppm between 2 or more points.

The GSD91 geoid model must be used with the NGSD91.BIN coefficient and NGSD91.SLV grid limit files together with the GSD91.EXE program file. Additional information about the GSD91 Geoid Model can be obtained from the Geodetic Survey Division (GSD) of Geomatics Canada at the following address:

Information Services            Tel:(613) 995-4410

Geodetic Survey Division     Fax:(613) 995-3215

615 Booth Street                 E-mail: information @geod.nrcan.gc.ca

Ottawa, Ontario                   Internet: http://www.geod.nrcan.gc.ca

K1A 0E9                             GOPHER:gopher.nrcan.gc.ca

The GSD91 Geoid Model is installed by copying the NGSD91.BIN and the NGSD91.SLV files together with the GSD91.EXE program file in any subdirectory. A new Geoid Model must be defined in LGO Coordinate Systems Management Group by creating the GSD91 Model using GRS 1980 Ellipsoid, Geodetic Coordinates and locating the GSD91.EXE program name and its subfolder on the PC as per Figure 1.

# Figure 1: GSD91 Definition in LGO

Following the proper definition of the GSD91 Geoid Model in LGO, the GSD91 Geoid Model must then be specified in new or existing Coordinate Systems based on the same Ellipsoid (GRS 1980) as per Figure 2.

# Figure 2: GSD91 Attachment in a LGO Coordinate System

Once the GSD91 Geoid Model is correctly attached to one or more Coordinate Systems, it can be used to compute Geoid Separations in LGO Projects based on these Coordinate Systems. Geoid Separation calculations are initiated by selecting Compute Geoid Separations from the Tools pull down menu. If the local Latitude and/or Longitude of Points are outside of the GSD91 coefficient file limits, an error message will be presented saying that Geoid Separation has failed. Confirmation of the proper calculation of Geoid Separations will be presented so that Orthometric Heights can be displayed together with local Ellipsoid Heights as per Figure 3.

Figure 3: Confirmation of proper Geoid Separation Calculations in a LGO Project