INSTRUCTIONS

FOR

HT2_0 GEOID MODEL

INSTALLATION AND OPERATION

IN

LGO

August 14, 2004

The HT2_0 Geoid Model is available from Geomatics Canada to determine Geoid Separations between the GRS 1980 geocentric ellipsoid based on the NAD83 (CSRS98) datum and CGVD28 vertical datum. The HT2_0 model is based on the new Canadian 2’ by 2’ CGG2000 gravimetric Model with additional space-based Vertical Transformations to produce new (absolute) Geoid Separations between NAD83(CSRS98) and CGVD28 datums. The “absolute” accuracy is typically +/- 5 cm in Southern Regions of Canada and about a few decimetres in Northern Regions. The “relative” accuracy is typically 1 centimetre + 1 ppm between 2 or more points. It is therefore possible to obtain GPS-derived Orthometric Heights compatible with the Vertical Bench Mark system (CGVD28) surrounding the area.

The HT2_0 geoid model can be used with one or more of the HT2_0 coefficient files: HT2_0.BYN for the entire Canada, HT2_0e.BYN for Eastern Canada, HT2_0n.BYN for Northern Canada, or HT2_0w.BYN for Western Canada together with the HT2_0byn.EXE program file. Additional information about the HT2_0 Geoid Model can be obtained from the Geodetic Survey Division (GSD) of Geomatics Canada (GPS-H V2.1 Package) at the following address:

Information Services            Tel:(613) 995-4410

Geodetic Survey Division     Fax:(613) 995-3215

615 Booth Street                 E-mail: information @geod.nrcan.gc.ca

Ottawa, Ontario                   Internet: http://www.geod.nrcan.gc.ca

K1A 0E9                             GOPHER:gopher.nrcan.gc.ca

The HT2_0 Geoid Model is installed by copying one or more of the BYN files (HT2_0, HT2_0e, HT2_0n, or HT2_0w.BYN) together with the HT2_0byn.EXE program file in any subdirectory. A new Geoid Model must be defined in LGO Coordinate Systems Management Group by creating the HT2_0 Model using GRS 1980 Ellipsoid, Geodetic Coordinates and locating the HT2_0byn.EXE program name and its subfolder on the PC as per Figure 1.

# Figure 1: HT2_0 Definition in LGO

Following the proper definition of the HT2_0 Geoid Model in LGO, the HT2_0 Geoid Model must then be specified in new or existing Coordinate Systems based on the same Ellipsoid (GRS 1980) as per Figure 2.

# Figure 2: HT2_0 Attachment in a LGO Coordinate System

Once the HT2_0 Geoid Model is correctly attached to one or more Coordinate Systems, it can be used to compute Geoid Separations in LGO Projects based on these Coordinate Systems. Geoid Separation calculations are initiated by selecting Compute Geoid Separations from the Tools pull down menu. If the local Latitude and/or Longitude of Points are outside of the HT2_0 coefficient files limits, an error message will be presented saying that Geoid Separation has failed. Confirmation of the proper calculation of Geoid Separations will be presented so that Orthometric Heights can be displayed together with local Ellipsoid Heights as per Figure 3.

Figure 3: Confirmation of proper Geoid Separation Calculations in a LGO Project