GEOID03 for Leica Geo Office

GEOID03 for LGO

January, 2004

Leica Geosystems Incorporated

1. INTRODUCTION:

The Geoid03 model can be used in Leica Geo Office (LGO) for geoid undulation calculations based on NAD83 Latitudes and Longitudes. The Geoid03 option consists of a single program, named GEOID03.EXE, with one or more grid coefficient file(s), named g2003u*.bin, corresponding to the different 8 areas of continental USA (CONUS). Position limits of the 8 CONUS areas are listed below:

g2003u01.bin       GEOID03 grid #1 for CONUS (40-58N, 111-130W)

g2003u02.bin       GEOID03 grid #2 for CONUS (40-58N, 094-113W)

g2003u03.bin       GEOID03 grid #3 for CONUS (40-58N, 077-096W)

g2003u04.bin       GEOID03 grid #4 for CONUS (40-58N, 060-079W)

g2003u05.bin       GEOID03 grid #5 for CONUS (24-42N, 111-130W)

g2003u06.bin       GEOID03 grid #6 for CONUS (24-42N, 094-113W)

g2003u07.bin       GEOID03 grid #7 for CONUS (24-42N, 077-096W)

g2003u08.bin       GEOID03 grid #8 for CONUS (24-42N, 060-079W)

It is under the user’s responsibility to obtain the right number and the adequate Geoid2003 Grid coefficient (g2003*.bin) file(s). The g2003u*.bin files must be the ones supplied by LEICA or must be previously converted to Intel binary format for the required areas. The original g2003u*.bin files can be converted to Intel binary format by using the XNTG program supplied by NGS with appropriate instructions for little endian binary conversion. It is expected that some of the Geoid2003 grid coefficient files could be updated from time to time. Additional information about Geoid03 model can be found in the g2003rme.txt file contained in GEOID03.ZIP.

2. INSTALLATION and OPERATION in LGO:

Decompress (pkunzip) GEOID03.ZIP file in any subdirectory existing on the current drive where LGO resides (C:\Geoid03 as an example). The GEOID03.ZIP file should contain the following files: GEOID03.EXE, GEOID03.DOC, G2003RME.TXT and one or more Geoid03 Grid Coefficient (g2003u*.bin) files as received from your LEICA representative or downloaded from NGS web site. Make sure that there is at least ONE Grid Coefficient file (g2003u*.bin) in that same subdirectory corresponding to your working area.

The GEOID03 Geoid Model is installed by copying the GEOID03EXE program file with one or more g2003u*.bin file(s) in any subdirectory. A new Geoid Model must be defined in LGO Coordinate Systems Management Group by creating the GEOID03 Model using GRS 1980 Ellipsoid, Geodetic Coordinates and locating the GEOID03.EXE program name and its subfolder on the PC as per Figure 1.

Figure 1: Definition of GEOID03 Model in LGO

Following the proper definition of the GEOID03 Geoid Model in LGO, the GEOID03 Geoid Model must then be specified in new or existing Coordinate Systems based on the same Ellipsoid (GRS 1980) as per Figure 2.

# Figure 2: GEOID03 Attachment in a LGO Coordinate System

Once the GEOID03 Geoid Model is correctly attached to one or more Coordinate Systems, it can be used to compute Geoid Separations in LGO Projects based on these Coordinate Systems. Geoid Separation calculations are initiated by selecting Compute Geoid Separations from the Tools pull down menu. If the local Latitude and/or Longitude of Points are outside of the GEOID03 coefficient files limits, an error message will be presented saying that Geoid Separation has failed. Confirmation of the proper calculation of Geoid Separations will be presented so that Orthometric Heights can be displayed together with local Ellipsoid Heights as per Figure 3.

Figure 3: Confirmation of proper Geoid Separation Calculations in LGO