GEOID99 for Leica Geo Office

GEOID99 for LGO

August, 2004

Leica Geosystems Incorporated

1. INTRODUCTION:

The Geoid99 model can be used in Leica Geo Office (LGO) for geoid undulation calculations based on NAD-83 Latitudes and Longitudes. The Geoid99 option consists of a single program, named GEOID99.EXE, with one or more grid coefficient file(s), named g1999*.bin, corresponding to different areas of continental USA (CONUS), Alaska, Hawaii, Puerto-Rico and Virgin Islands territories. Position limits of the 8 CONUS, 4 Alaska, 1 Hawaii and 1 Puerto-Rico/Virgin Islands areas are listed below:

g1999u01.bin       GEOID99 grid #1 for CONUS (40-58N, 111-130W)

g1999u02.bin       GEOID99 grid #2 for CONUS (40-58N, 094-113W)

g1999u03.bin       GEOID99 grid #3 for CONUS (40-58N, 077-096W)

g1999u04.bin       GEOID99 grid #4 for CONUS (40-58N, 060-079W)

g1999u05.bin       GEOID99 grid #5 for CONUS (24-42N, 111-130W)

g1999u06.bin       GEOID99 grid #6 for CONUS (24-42N, 094-113W)

g1999u07.bin       GEOID99 grid #7 for CONUS (24-42N, 077-096W)

g1999u08.bin       GEOID99 grid #8 for CONUS (24-42N, 060-079W)

g1999a01.bin       GEOID99 grid #1 for Alaska (60-72N, 156-188W)

g1999a02.bin       GEOID99 grid #2 for Alaska (60-72N, 126-158W)

g1999a03.bin       GEOID99 grid #3 for Alaska (49-61N, 156-188W)

g1999a04.bin       GEOID99 grid #4 for Alaska (49-61N, 126-158W)

g1999h01.bin       GEOID99 grid #1 for Hawaii (18-24N, 154-161W)

g1999p01.bin       GEOID99 grid #1 for P-R/VI (15-21N, 064-069W)

It is under the user’s responsibility to obtain the right number and the adequate Geoid99 Grid coefficient (g1999*.bin) file(s) for the required territory(ies). A maximum of 14 Grid coefficient files can be opened by the program. It is expected that some of the Geoid99 grid coefficient files will be updated from time to time. Additional information about Geoid99 model can be found in the g1999rme.txt file contained in GEOID99.ZIP.

2. INSTALLATION and OPERATION in LGO:

Decompress (pkunzip) GEOID99.ZIP file in any subdirectory existing on the current hard disk drive (C:\GEOID99 as an example). The GEOID99.ZIP file should contain the following files: GEOID99.EXE, GEOID99.DOC, G2003RME.TXT and one or more GEOID99 Grid Coefficient (g1999*.bin) files as received from your LEICA representative or downloaded from NGS web site. Make sure that there is at least ONE Grid Coefficient file (g1999*.bin) in that same subdirectory corresponding to your working area.

The Geoid Model is installed by copying the GEOID99EXE program file with one or more g1999*.bin file(s) in any subdirectory. A new Geoid Model must be defined in LGO Coordinate Systems Management Group by creating the GEOID99 Model using GRS 1980 Ellipsoid, Geodetic Coordinates and locating the GEOID99.EXE program name and its subfolder on the PC as per Figure 1.

Figure 1: Definition of GEOID99 Model in LGO

Following the proper definition of the GEOID99 Geoid Model in LGO, the GEOID99 Geoid Model must then be specified in new or existing Coordinate Systems based on the same Ellipsoid (GRS 1980) as per Figure 2.

# Figure 2: GEOID99 Attachment in a LGO Coordinate System

Once the GEOID99 Geoid Model is correctly attached to one or more Coordinate Systems, it can be used to compute Geoid Separations in LGO Projects based on these Coordinate Systems. Geoid Separation calculations are initiated by selecting Compute Geoid Separations from the Tools pull down menu. If the local Latitude and/or Longitude of Points are outside of the GEOID99 coefficient files limits, an error message will be presented saying that Geoid Separation has failed. Confirmation of the proper calculation of Geoid Separations will be presented so that Orthometric Heights can be displayed together with local Ellipsoid Heights as per Figure 3.

Figure 3: Confirmation of proper Geoid Separation Calculations in LGO